ESP32 WROOM U32 Wifi+Bluetooth Configuration

Producer : Yıldırım Elektronik

- Özgün Çevik

- Hızır Arda Can (Stajer 07-08.2025)

Date: 18.07.2025

## End-to-End Guide: Building & Flashing ESP-AT Firmware

This document walks you through setting up your development environment, configuring the AT firmware, building it, and flashing it onto an ESP32-WROOM-32U module, from zero to final.

### 1. Prerequisites

- Ubuntu 22.04 (or similar Linux)

From: https://ubuntu.com/download

- Virtual Machine or native install with at least 4 GB RAM, 20 GB free disk

From: https://www.virtualbox.org/

- USB-to-UART adapter and jumper wires

IMPORTANT Note: Use UART1 for connection from ESP32 WROOM 32U to STM32 Board

For ESP: RX = 16 , TX=	17	.

For STM32, it doesn’t matter, but our code configures with respect to UART6, so RX = PC7, TX PC6.

- ESP32-WROOM-32U development board

### 🔧 How to Install Core Packages and Set Serial Access on Ubuntu (Step-by-Step)

#### 📌 Step 1: Open the Terminal

There are two easy ways to open the Terminal.

##### 🔸 Method 1: Using the Mouse (Right Click)

- Move your mouse to an empty area on your desktop (no icons).

- Right-click with your mouse.

- From the menu that appears, click “Open Terminal”.

##### 🔸 Method 2: From the Application Menu

- Click the "Show Applications" button at the bottom-left (looks like a grid of 9 dots).

- In the search bar, type “Terminal”.

- Click the Terminal icon to open it

##### 🔸 Method 3: Use Shortcut

- CTRL+ALT+T

#### 📌 Step 2: Update the Package List (Optional)

Now we will update your system to make sure you get the latest package versions.

💡 Note:
 If you just installed Ubuntu for the first time, this step is usually not necessary. But if Ubuntu has been installed for a while, it's a good idea to run this to refresh the list of available software.

To update the package list, type (or copy and paste) the following command into the terminal, then press Enter:

sudo apt update

- It will ask for your password. Type your computer password (you won't see anything as you type — that's normal), then press Enter again.

You can skip this step if your system is new, but it’s still recommended to do it if you're not sure.

#### 📌 Step 3: Install Core Development Packages

Next, install the required tools for development.

Copy and paste this long command into the terminal, then press Enter:

sudo apt install git wget flex bison gperf python3 python3-pip python3-setuptools cmake ninja-build ccache libffi-dev libssl-dev dfu-util

- If it asks "Do you want to continue? [Y/n]", press Y and then Enter.

#### 📌 Step 4: Allow Serial Port Access (Dialout Group)

This step allows your computer to talk to devices through USB or serial ports (like ESP32 or Arduino boards).

Copy and paste the following command into the terminal and press Enter:

sudo usermod -a -G dialout $USER

- OR you can write your user name instead of “$USER”

Ex: sudo usermod -a -G dialout ozgun

#### 📌 Step 5: Log Out and Log Back In

To apply the group change:

- Click on your profile picture at the top-right corner of the screen.

- Choose "Log Out".

- After logging out, log back in with your username and password.

💡 Note: Restarting the computer also works and applies the changes.

### 2. Clone & Install ESP-IDF

Get the correct IDF version (v5.0.1):

cd ~

git clone -b v5.0.1 --recursive https://github.com/espressif/esp-idf.git

Install tools & Python venv:
cd ~/esp-idf

./install.sh

Possible Errors:

Error 1: Wrong Directory:

bash: ./install.sh: No such file or directory

Solution 1 :

Navigate to the esp-idf directory
 Open your file explorer and find the folder named esp-idf.

Open Terminal in that directory

- Left-click on the esp-idf folder or an empty space inside it to select it.

- Then right-click (or use the menu) and choose Open in Terminal.

This will open a terminal window with the current path set to the esp-idf folder.

Run the installation script
 In the terminal, type the following command and press Enter:

./install.sh

Export environment:
. ./export.sh

After running the screen like this.

Error 2: A fatal error occurred: No module named 'xlrd'

Solution 1 :

Run :

pip install xlrd

OR

pip3 install xlrd

According to your Python version.

Try to run again:

./install.sh

### 3. Clone & Checkout ESP-AT

Clone the AT firmware: (It can take some time according to your computer and internet) 
cd ~

git clone https://github.com/espressif/esp-at.git

cd esp-at

Checkout desired AT release (v3.2.0.0):
git checkout v3.2.0.0

git submodule update --init --recursive

Verify the ESP-IDF submodule points to v4.4 (for AT v3.2.0.0):
cd esp-idf

git checkout v5.0.1

. ./export.sh

cd ..

### 4. Build AT Firmware (build)

Run:

./build.py build

#### A) Select your ESP Model

- We selected Platform ESP32; therefore, write 1.

- Also  We selected ESP32 WROOM 32; therefore, write 1.

IMPORTANT: Choose silence mode as 1, Silence Mode configuration is not accessible after this comment. It can cause IRAM Overflow.

- If it is stuck in the wifi library, wait (If you have a bad pc, open 1 episode series :D ); this part may take a while. Don’t Panic.

After build you can directly flash (As a default configuration, just WIFI is enabled) or you can follow step 4.

### 4. Configure AT Firmware (menuconfig)

Run:

./build.py menuconfig

- Go to Component config
 Navigate into the Component config menu.

- Go to Bluetooth
 Inside Component config, select Bluetooth.

- Check Bluetooth has a *
 Make sure Bluetooth is selected (marked with an asterisk *).

- Go to Host
 Inside Bluetooth, enter the Host submenu.

- Check Bluedroid-dualmode has a *
 Verify that bluedroid-dualmode is enabled (marked with *).

- Exit from Host
 Exit the Host submenu.

- Go to Bluedroid options
 Enter the Bluedroid options submenu.

- Disable Bluetooth Low Energy
 Find the Bluetooth Low Energy option and disable it (remove the *).

- Enable Classic Bluetooth
 Make sure Classic Bluetooth is enabled (marked with *).

Under Classic Bluetooth, enable SPP (Serial Port Profile).

- Press “S” and save the configuration
 Press the S key to save your configuration changes.

AT Configuration

- Return to Component config menu.

- Enter the AT menu.

- Enable AT Base command support (has *).

- Enable AT User command support (has *).

- Enable AT+USERWKMCU command support (has *).

- Enable AT WiFi command support (has *).

- Enable AT Net command support (has *).

- Enable AT Ping command support (has *).

- Enable AT BT SPP command support (has *).

- Enable AT Signaling Test command support (has *).

- Enable AT OTA command support (has *).

- Save and Exit

### 5. Build & Check Size

#### 5.1 Build firmware:

./build.py build

#### 5.2 Check memory usage:

-OPTIONAL

./build.py size-components

Ensure IRAM usage is under ~64 KiB with no overflow.

### 6. Flash & Monitor

Connect your ESP32 board via USB-to-UART. Then:

idf.py -p /dev/ttyUSB0 -b 115200 flash monitor

- If permission denied: sudo usermod -a -G dialout $USER, log out/in.

- If you want to just flash use this code :

idf.py -p /dev/ttyUSB0 -b 115200 flash monitor

Monitor controls:

- Exit: Ctrl-] (US) (Ubuntu has US keyboard so , press CTRL+Ü)

Possible Errors:

Error 1: USB port not connected.

Solution:

To resolve the error, right-click on the USB icon located at the bottom right corner of the screen. In the window that appears, locate the device and check the box next to its name to enable it. Then, run the command again.

idf.py -p /dev/ttyUSB0 -b 115200 flash monitor

### 7. Verify AT Firmware

#### 7.1 AT Firmware Server Wifi

AT
 Basic communication check — verifies if ESP32 is responding.
 👉 Expected response: OK

AT+RST
 Resets the module to clear previous states.
 👉 Expected response: OK (followed by reboot messages)

AT+CWMODE=3
 Set Wi-Fi mode to AP+Station mode.
 👉 Expected response: OK

AT+CWSAP="Ozgun","123456789",6,3,4,0
 Configure Access Point SSID, password, channel, encryption, max connections, and visibility.
 👉 Expected response: OK

AT+CIPMUX=1
 Enable multiple TCP connections.
 👉 Expected response: OK

AT+CIPSERVER=1,5000
 Start TCP server on port 5000.
 👉 Expected response: OK

#### 7.1 AT Firmware WIFI

- AT
 Basic communication check — verifies if ESP32 is responding.
 👉 Expected response: OK

- AT+GMR
 Displays the firmware and SDK version information.
 👉 Expected response: Firmware version details.

- AT+BTINIT=1
 Initializes the Bluetooth stack.
 👉 Expected response: OK

- AT+BTINIT?
 Checks whether the Bluetooth stack is initialized.
 👉 Expected response: +BTINIT:1

- AT+BTNAME="ESP32_Bluetooth"
 Sets a visible Bluetooth name for the device.
 👉 Expected response: OK

- AT+BTSCANMODE=2
 Makes the ESP32 discoverable and connectable.
 👉 2 → Discoverable and Connectable
 👉 Expected response: OK

- AT+BTSCAN=1,1
 Starts Bluetooth device scanning (inquiry scan).
 👉 Expected response: Detected devices will be listed as +BTSCAN: ...

- AT+BTSPPCONN=<mac_address>
 Connects to a Bluetooth device using its MAC address.
 👉 Expected response: OK or CONNECT

- AT+BTSPPSEND=0,5
 Begins transmission of 5 bytes over the SPP connection (Link ID 0).
 👉 This is typically followed by sending the string HELLO.